Street: 	The Quay

Building: 	Portsmouth Hoy Public House

Grid ref: 	SZ 0090  SE 2/238		28.5.74

Grade:	II		GV

Description:	c18 façade.  Stucco, with heavy, corbelled eaves cornice, low coped blocking course and old tiled roof with stone verges.  Tall end chimneys supported by iron braces from ridge.  Two storeys and skylights; 3 windows, unbarred sashes under segmental arches with keystones.  Ground floor has modern glazing, windows late c19, with plain architrave surrounds under bracketed labels each side of centre door with segmental arch and keystone.  Attic stair has late c18 balustrade (RCHM)

Planning History:1984 – Erect extension at side and access to 1st floor